| REV. | Description |  |  |  |  |  | Date |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ##### 00 | SPEC ISSUE (NEW MODEL) ADP-400GR AAA |  |  |  |  |  | 05/27’22 |  |
| 01 | 102A-229084 Add Model：ADP-400GR AAB, ADP-400GR AAC |  |  |  |  |  | 09/16’22 |  |
| 02 | 102A-22B236 1. Change Item 10-5. Power noise (Condition & Specification) |  |  |  |  |  | 12/01’22 |  |
| ##### 03 | 102A-22C035 Add Model：ADP-400GR AAD |  |  |  |  |  | 12/12’22 |  |
| ##### 04 | 102A-22C108 1. UPDATE ITEM 11.2 SHOCK SPEC |  |  |  |  |  | 12/20’22 |  |
| ##### 05 | 102A-231044 1. UPDATE ITEM 11.2 SHOCK SPEC |  |  |  |  |  | 01/10’23 |  |
| ##### 06 | 102A-22C106 Add Model：ADP-400GR AA |  |  |  |  |  | 01/18’23 |  |
| ##### 07 | 102A-232105 Add Model：ADP-400GR AAE, ADP-400GR AAF |  |  |  |  |  | 02/15’23 |  |
| ##### 08 | 102A-235049 Add Model：ADP-400GR AB, ADP-400GR ABA, ADP-400GR ABB, ADP-400GR ABC |  |  |  |  |  | 05/05’23 |  |
| ##### 09 | 102A-235308 1. CR-BOX (L-G/N-G) surge criteria change from 6KV to 8KV (no function error, no latch off, no damage) ; 8KV to 10KV (can damage, no quality accident.) |  |  |  |  |  | 05/31’23 |  |
| ##### 10 | 102A-238047 Add Model：ADP-400GR AB88 |  |  |  |  |  | 08/04’23 |  |
| ##### 11 | 102A-244108 1. ITEM 9.3 重量變更由 "525" 變更為 "490g+/-10g" |  |  |  |  |  | 04/11’24 |  |
| ##### 12 | 102A-244263 Add Model：ADP-400GR AC |  |  |  |  |  | 04/23’24 |  |
|  |  |  | 台達電子工業股份有限公司 DELTA ELECTRONICS, INC. |  |  | DESCRIPTION : 電氣規格 (Electrical Specification) |  |  |
| THESE DRAWINGS AND SPECIFICATIONS ARE THE PROPERTY OF DELTA ELECTRONICS, INC. AND SHALL NOT BE REPRODUCED OR USED AS THE BASIS FOR THE MANUFACTURE OR SELL OF APPARATUSES OR DEVICES WITHOUT PERMISSION. |  |  |  |  |  | MODEL NO. : ADP-400GR A SERIES |  |  |
| Date |  | Drawn |  | Design (EE) | Design (ME) | DOCUMENT NAME. : ES-400GR A SERIES |  | REV. |
| 04/23’24 |  | 李亭芳 |  | 楊曜廷 | 李昱緯 |  |  | 12 |

FRAME Name:DF-PSLA4V-2R01.DOC                            SHEET  1  OF  17

MODEL LIST

| - ADP-400GR AA | - ADP-400GR AAA | - ADP-400GR AAB | - ADP-400GR AAC | - ADP-400GR AAD |
| --- | --- | --- | --- | --- |
| - ADP-400GR AAE | - ADP-400GR AAF | - ADP-400GR AB | - ADP-400GR ABA | - ADP-400GR ABB |
| - ADP-400GR ABC | - ADP-400GR AB88 | - ADP-400GR AC |  |  |

1.Scope

- This specification applies to the switching regulator used for Class II type products.

2.Specification

- Refer to the following

3.Applicable Standard

3-1. Safety specifications

| - International | - IEC 60950-1:2005 +A1:2009 + A2:2013 |
| --- | --- |
|  | - IEC 62368-1:2014 (Second Edition) |
|  | - IEC 62368-1:2018 (Third Edition) |
| - United States | - UL 62368-1, 2nd Ed, 2014-12-01 |
|  | - UL 62368-1, 3rd Ed, 2019-12-13 |
| - Canada | - CAN/CSA C22.2 No.62368-1-14, 2nd Ed |
|  | - CAN/CSA C22.2 No.62368-1-19, 3rd Ed |
| - Europe | - EN 62368-1:2014 (Second Edition) + A11:2017 |
|  | - EN 62368-1:2020 (Third Edition) + A11:2020 |
| - Korea | - K60950-1 (2011-12) |
| - China | - CQC GB4943.1-2011 (Safety) |

- 3-2. EMC

| EMC | Item | Specifications and Test Conditions |
| --- | --- | --- |
| EMI | Conduction (雜音端子電壓) | VCCI-B, FCC Part15 B EN55032, KS C 9832:2019, CISPR32, CISPR35 |
|  | Radiation (不要輻射) | VCCI-B, FCC Part15 B EN55032, KS C 9832:2019, CISPR32, CISPR35 |
|  | Harmonic (電源高調坡) | EN 61000-3-2 |
|  | Flicker (電源閃爍) | EN 61000-3-3 |
| EMS | Electrostatic Discharge (靜電氣放電) | IEC61000-4-2 Test Probe: 150pF / 330 ohm Air : ~ +/- 8kV Contact : ~ +/-6kV No function error, no latch off, no damage. |
|  | RS (放射無線周波數磁界) | IEC61000-4-3 3V/m AM modulation 1kHz 80% 80~10000MHz Sweep rate: 1.5x10-3 decade/s No function error, no latch off, no damage. |
|  | EFT/B (快速暫態雜訊) | IEC61000-4-4 1KV No function error, no latch off, no damage. |
|  | Surge (雷擊) | IEC61000-4-5 Common : +/-4KV Normal : +/-2KV Phase + : 0oC, 90oC Phase - : 180oC, 270oC No function error, no latch off, no damage. |
|  | CS (傳導性妨害) | EC61000-4-6 3V AM modulation 1kHz 80％ 0.15～80MHz Sweep rate:1.5x10-3 decade/s No function error, no latch off, no damage. |
|  | Immunity (電源周波數磁界) | IEC61000-4-8 1A/m x,y,z direction No function error, no latch off, no damage. |
|  | AC dip (電源電壓變動) | IEC61000-4-11 Above 95%, reduction,0.5 cycle No function error, no latch off, no damage. (240Vac12Vac) & (100Vac5Vac) (Performance Criterion B) 30% reduction, 25 cycle (240Vac  168Vac) & (100Vac 70Vac) After the test, AC OFF then ON , then unit should be able to start again and no function error, no latch, no damage. (Performance Criterion C) Above 95%, reduction,250 cycle (240Vac 12Vac)&(100Vac5Vac) After thet test, AC OFF then ON , then unit should be able to start again and no function error, no latch off, no damage.(Performance Criterion C). |

| 4. Input characteristics |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Item | Condition |  | Specification |
| 1 | Rated input voltage | - |  | 100 ~ 240 Vac |
| 2 | Input voltage range | - |  | 85 ~ 276 Vac |
| 3 | Input current | 12V/31A @ 100Vac |  | Less than 4.5A |
| 4 | Rated input frequency | - |  | 50Hz / 60Hz |
| 5 | Input frequency | - |  | 47 ~ 63 Hz |
| 6 | Input inrush current | 85 ~ 276 Vac Rated load (Test with 3KV AC source). | Ta= 25oC Cold start | Less than 140A |
|  |  |  | Ta= 40oC Repeat AC ON<=>OFF 12V 0A | Less than 180A/2msec 以下 |

Safety certification

100-127V~4.5A

200-240V~2.3A

5. Output characteristics

5-1. Output System:

5-2 Output Characteristics:

Below output capacitance (smallest capacity) should be satisfied

12V  : 22uF (Ceramic capacitor)

| Output | Output Voltage |  | Output Current |  |  | Ripple & Nioise (at rated load) |
| --- | --- | --- | --- | --- | --- | --- |
|  | Rated | Tolerance | Peak | Rated | Min. | Ta= -5~40oC |
| 12V | 12V | + 5%,- 4% | 50.25A/30msec | 31A | 0A | 150mVp-p,max |

| Output | OCP | OVP |
| --- | --- | --- |
| 12V | 50.25 ~ 62.8A | 15.6V,max |

Power ON/OFF (Sequence and Timing)

| Item | Condition | Specification |
| --- | --- | --- |
| t_on | AC ON to 12V turn on time | output voltage can be regulated to 12V+5%, -4% within 2S. |
| t_off | 12V output voltage from 100% to 10%. | 12V output voltage from 100% to 10%. |
| t_r | 0V to 12V rise time | 12V output voltage from 5% to 95% within 2~20msec. |
| t_dur 1 | Rate input Load : 20A or less | Hold up time. Output voltage should keep in range of rated voltage more than 30ms. |
| t_9V_keep | After then AC OFF ,12V during hold up time(t_dur 1). And then 12V output voltage is set to 9V. When load is more than 200(mA). When load is less than 1A in 9V output mode. PSU should keep output for 1s or more. And if PSU is able to work normally by using internal remaining power, output should keep 9[V]（+5%/-4%） as long as possible. . Undershoot voltage should not be less than 7[V] in output mode change situation between 12V mode and 9V mode. | 9V keeping time while 12V output voltage is set to 9V output mode after AC OFF. |

6. Other characteristics:

|  | Item | Condition and Specification |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | Over / Undershoot | Input range: 85 ~ 276 Vac Output voltage should be within 10% of rated output voltage. |  |  |  |
| 2 | Start with capacitive load | Input range: 85 ~ 276 Vac Output should be in parallel with below capacitor : 12V : 12000uF The unit should be able to start stably. |  |  |  |
| 3 | Protective function | OCP |  | The 12V has been detected abnormal (12V OC/OV/OT/UV), 12V output should be shut down immediately. |  |
|  |  | OVP |  |  |  |
|  |  | OTP |  |  |  |
|  |  | Protection functional operation (Signal and power output) |  |  |  |
| 4 | Recovery from protection | Remove latch function after the AC OFF and within 3 minutes. |  |  |  |
| 5 | Transient response | 12V | Input range: 85 ~ 276 Vac [ 12V load change ] Change range : 100～40[%], 90～30[%], 80～20[%], 70～10[%] of rated current Change frequency : 10[Hz]～20[kHz]. Change rate : 1.0A/use |  | Output voltage should be requlated within 12V +5% -4%. |
| 6. | Operating load range under natural air cooling | Load Condition: 7[A] or less under 25℃ / 4.5[A] or less under 40℃ Required Performance: Must operate normally without OTP. 。 Not exceed parts temperature rating. No abnormal interference to "end-product" operation. If necessary, set appropriate temperature de-rating which decided to satisfy the above criterion after mutual agreement. |  |  |  |

| 7. Common specifications |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Item |  |  | Condition | Specification |
| 1 | Switching frequency |  |  | 100Vac input. Rated load. | - |
| 2 | Power factor |  |  | - | IEC61000-3-2. |
| 3 | Life |  |  | Rated input voltage. Ambient temperature, constant humidity. | 70,000Hr or more with thermal test BOX. => 20,000Hr, 12V rated load (FAN cooling). => 50,000Hr, 12V 3A (w/o Fan). |
| 4. | Efficiency | Light load w/o Fan |  | Rated input voltage, load (0A). | <0.17W |
|  |  |  |  | Rated input voltage, load (0.01A). | 37.8 |
|  |  |  |  | Rated input voltage, load (0.025A). | 61.5 |
|  |  |  |  | Rated input voltage, load (0.05A). | 66.4 |
|  |  |  |  | Rated input voltage, load (0.1A). | 72.4 |
|  |  |  |  | Rated input voltage, load (0.2A). | 76.7 |
|  |  |  |  | Rated input voltage, load (0.3A). | 82.0 |
|  |  |  |  | Rated input voltage, load (0.4A). | 83.0 |
|  |  |  |  | Rated input voltage, load (0.6A). | 84.0 |
|  |  |  |  | Rated input voltage, load (1A). | 85.0 |
|  |  | Normal load With Fan |  | Rated input voltage, load (10A). | 89.0 |
|  |  |  |  | Rated input voltage, load (20A). | 88.0 |
|  |  |  |  | Rated input voltage, load (31A). | 85.5 |
| 5 | Parts temperature |  |  | Rated input voltage. Thermal test BOX. Ta=40oC | There is enough de-rating of temperature for all component. No component over derating before OTP trigger. |
| 6 | Hold up time (瞬停保証時間) |  | t_dur1 | Rated input voltage 12V: 20A | 50Hz 1.5 cycle (=30ms). Output voltage shall be satisfy electric characteristic. |
| 7. | Leakage current |  |  | Normal temp and humidity Rated input | 100uA or less |
| 8 | Input discharge time |  |  | Input 121V or less, from AC power OFF to 37% rated. | 1 sec or less. |
|  |  |  |  | Input 122V or more, from AC power OFF to 45V. | 1 sec or less. |
| 9 | 入力片切test |  |  | AC ON Line OPENLine SHORT. AC ONNeutral OPENNeutral SHORT. | Same as AC ON AC OFF AC ON Same as AC ON AC OFF AC ON |
| 10 | Overvoltage apply test |  |  | 400Vac input / 2sec. | No safety hazard. |
| 11 | Dielectric strength |  |  | Input between primary and secondary. QA (sampling) : 3.0kVac 50/60Hz 60sec Production line (all): 3.1kVac 50/60Hz 3sec | IEC60950-1 item 5.2 Sense current : 10mA or less. Without damage to parts. |
| 12 | Insulation resistance |  |  | 500 Vdc input between primary and secondary. | 10 Mohm or more. |
| 13 | Insulation distance |  |  | - | Should be met the safety requirement and add external 0.5mm for SIE request. |

| 8. Environmental |  |  |
| --- | --- | --- |
|  | Item | Condition & Specification |
| 1 | Operation temperature and humidity range | -5 ~ +40oC (humidity 20~90% Rh) |
| 2 | Storage temperature and humidity range | -40 ~ +80oC (humidity 10~90% Rh) |
| 3 | High humidity operation | Treatment : Storage for preprocessing before Hi temperature operation +35oC/90% Rh Duration 8h Electric characteristic shall be satisfied. |
| 4 | Low temperature operation | Treatment : Storage for preprocessing before Low temperature operation -5oC Duration 8hr Electric characteristic shall be satisfied. |
| 5 | Continuous operation under high temperature and high humidity | Condition of continuous operation +40oC/90% Rh 1000Hr operation. Electric characteristic shall be satisfied. |
| 6. | Continuous operation under room temperature and room humidity | Condition of continuous operation +25oC/30% Rh Duration 48h Electric characteristic shall be satisfied. |
| 7. | Continuous operation under low temperature | Condition of continuous operation -10oC Duration 48hr Electric characteristic shall be satisfied. |
| 8 | Continuous operation under low humidity | Condition of continuous operation +20oC/20% Rh Duration 48hr Electric characteristic shall be satisfied. |
| 9 | Low temperature ON/OFF | Rated input voltage±10%. Rated load. Electric characteristic shall be satisfied. Testing follow below temperature curve: |
| 10 | High temperature and high humidity storage | After put the unit with max. storage +80oC/90% Rh , duration 100Hr, then change the condition to ambient temperature & humidity 1Hr, and then test it. Electric characteristic shall be satisfied. |
| 11 | Low temperature storage | After put the unit with min. storage -40oC, duration 100Hr,, then change the condition to ambient temperature 1Hr, and then test it. Electric characteristic shall be satisfied. |
| 12. | Humidity resistance | Treatment 1 : High temperature and high humidity storage Temperature: 40±2℃ Humidity 90~95% Duration: 48 hour Treatment 2 : Standard atmospheric storage Wipe moisture after treatment 1 , and then storage at the following condition Temperature / Humidity : Room temperature and normal humidity. Duration : 30min Criteria after the above test : Dielectric strength and insulation resistance must be satisfied. |
| 13 | Thermal shock | Treatment : thermal shock (non-operation storage) -40℃~80℃ / 100(Cyc) Electric characteristic shall be satisfied and without distinct damage in appearance. |
| 14 | Heat cycle | Treatment : thermal shock (non-operation storage) -40℃~80℃ / 90 % RH Electric characteristic shall be satisfied and without distinct damage in appearance. |

| 9. Other |  |  |
| --- | --- | --- |
|  | Item | Condition & Specification |
| 1 | Appearance | There is no adhesion of the foreign material and the dirty fingerprint. In addition, when producing it depends on the oil and the like which is used, there is no stench of the product. |
| 2 | Adhesive | Chloroprene is designated as the main component, do not use the adhesive. |
| 3 | Mass | 490g+/-10g |
| 4 | Noise | - |

| 10. 耐久性能 |  |  |  |
| --- | --- | --- | --- |
|  | Item | Condition & Specification |  |
| 1 | Output open / short | 12V Rated input voltage. After output short 1sec, AC off (disable latch mode) then re-start as 1cycle, this 100 cycles after doing electric characteristic shall be satisfied. (將輸出端子Short 一秒後, AC OFF (Latch解除) 再切入AC 以此為lcycle, 實行100 cycle後, 滿足電器特性 。) |  |
| 2 | Power ON/OFF (Secondary) | Rated input voltage +/-10%. 12V: Rated load (resistor) Inside thermal test BOX,FAN cooling | The secondary ACDC_STBY control the ON/OFF shall be done 100,000 times. (5sec/ON, 5sec/OFF) Electric characteristic shall be satisfied. |
| 3 | O/S test | Input voltage : 90V & 264V Output load : Rated load. Open & Short parts: Transistor and IC: Combination between all terminals Diode: Between A-K Transformer & chock: Coil Capacitance: Between terminals When AC input turn on state, to do components open/short test. | No ignition or smoke or damage. |
| 4 | Electrostatic Discharge | IEC61000-4-2 Rated input voltage. Rated load. Test probe: 150pF / 330 ohm. Each 5 times. Discharge location: [Air] . Any surface hand can touch except the air intake vent. [Indirect] [Contact] . Any metal hand can touch. . power & signal (input/output). | Air : +/- 12KV: no function error. +/- 14KV: no latch off, no damage. +/- 15KV: no quality accident. |
|  |  |  | Indirect : +/- 8KV: no function error, no latch off, no damage. +/- 15KV: no quality accident. |
|  |  |  | Contact : +/- 12KV: no function error, no latch off, no damage. +/- 15KV: no quality accident. |
| 5 | Power noise | Rated input voltage, rated load. Noise frequency：30～100Hz Noise level ：±200[V]～1[kV]（200[V] step） Pulse width ：100[nsec] and 1000[nsec] Input mode ：Normal mode for Live and Normal mode for Neutral Input duration ：1[min] or more (for each voltage condition） | No function error, no latch off, no damage. |
| 6 | Lighting surge 雷擊 | Rated input voltage, rated load. IEC1000-4-5 with CR box (13 ohm + 9uF) Phase +: 0o, 90o. Phase -: 180o, 270o. Each 5 times. | L-N/N-L +/- 6KV: no function error, no latch off, no damage. +/- 8KV: cam damage, no quality accident. L-G/N-G +/- 8KV: no function error, no latch off, no damage. +/- 10KV:can damage, no quality accident. |

11. MECHANICAL

| Item |  | Judgement | Condition |
| --- | --- | --- | --- |
| 1 | Vibration(1) | Electric characteristic shall be satisfied. And no remarkable abnormal occurrence on the appearance and construction. Parts and assembly with customer system must also be satisfied. | Non-operating/Random: The entire frequency range from 7Hz(0.822 g2 /Hz) to 30Hz(0.192 g2 /Hz) and return to 7Hz(0.822 g2 /Hz) shall be a transverse in 5 min. Three axis (X,Y,Z direction) shall be applied 20minutes acceleration: 23.5 m/s2 (2.85Grms) |
|  | Vibration (2) | 判定基準： 1. 沒有異音（於測試過程or測試後, 不可有測試設備以外的異音） 2. 電気特性要満足。外観與構造沒有明顯異常 | 振動数：50Hz ～ 130Hz 加速度：2 [m/s^2] 振動Type：Sweep 方向＆時間：X,Y,Z 10分間 |
| 2 | Shock | Electric characteristic shall be satisfied. And no remarkable abnormal occurrence on the appearance and construction. Parts and assembly with customer system must also be satisfied. | 1.Non-operating/half-sine Three axis (X,Y,Z direction) shall be applied in both directions of mutually perpendicular axis 1 times (a total of 6 times) for table a. Sample : 1pcs 累積保証. 落下面; G; Duration. 1面; 250; 5. 2面; 250; 5. 3面; 250; 5. 4面; 250; 5. 5面; 250; 5. 6面; 250; 5. 2.Non-operating/half-sine A directions for a sample, G & Duration value follow table b Total sample : 6pcs 単面保証. 落下面; G; Duration. 1面; 500; 2. 2面; 500; 2. 3面; 650; 2. 4面; 400; 2.5. 5面; 650; 2. 6面; 500; 2. |
| 3 | Drop I | 1. No component broken.. 2. No PWB copper pad peeling and broken 3. Hi-pot test pass with specific condition. ( AC 3300V/1min ) 4. No soldering crack. 6. ATS function Pass. | 測試前,Case 請用螺絲鎖附在系統配合孔固定case 1.Drop TIMES: 5 times for every surface (six side),total 30 times 2. Test surface material : The concrete 3.Drop height: 10cm. |
| 4 | Drop II | 1. No component broken.. 2. No PWB copper pad peeling and broken 3. Hi-pot test pass with specific condition. ( AC 3300V/1min ) 4. No soldering crack. . 5. ATS function Pass. | 測試前,Case 請用螺絲鎖附在系統配合孔固定case 1.Drop TIMES: 1 times for every surface (six side),total 6 times 2. Test surface material : The concrete 3.Drop height: 45cm. |

12. Packing test :

12.1 Drop test condition:

<1>.Drop height:80.0CM

<2>.Impact surface: concrete floor

<3>.Drop times: 1 side (Bottom side),total 10 times

Criteria:

The sample has no electric characteristic issue and without

distinct damage in appearance.

12.2 Vibration test condition:

<1>.Wave: Random

<2>.Acceleration: 1.146Grms

<3>.Frequency: 5-200Hz

<4>.Duration:30min/side

<5>.Orientations: each axis total 3 times

Criteria:

The sample has no electric characteristic issue and without distinct damage in appearance.

12.3 Pallet Test

<1>. Container ( Truck ) Shipping test

- Test Distance: Manufacturing factory to ODM including return distance

- Test Item: Full packaged pallet

- Criteria: 1) PSU No Damage 2) Shipping material can’t have structural damage. Minor damage is accepted. 3) No any dust, ash or condensation water inside tray or PSU.

<2>. Sea Transportation Test

- Test Distance: Manufacturing factory to TW DEIC with sea transportation

- Test Item: Full packaged pallet

- Criteria: 1) PSU No Damage 2) Shipping material can’t have structural damage. Minor damage is accepted. 3) No any dust, ash or condensation water inside tray or PSU.

13. For safety

13.1 Altitude : 2000m

13.2 Thermal test setup

14. CN101

| Pin_1 | Output (+) |  |
| --- | --- | --- |
| Pin_2 | Output (+) |  |
| Pin_3 | Output (-) |  |
| Pin_4 | Output (-) |  |

15. CFM: Burn-in air flow 8.3CFM(流經電源內部)

Product  Ingress protection(IP) rating: Not requirement

Product Application: Game Consoles